# Health and Safety Coordination Plan

Sherwood Views

Project Name

General Information

| ADDRESS |  | COMPANY NAME |  |
| --- | --- | --- | --- |
| 22 Iredale Promenade, Sunbury |  | Jackson Dwellings |  |
| POSITION | NAME |  | CONTACT NUMBER |
| Principal Contractor | Cameron Jackson |  | +61412286008 |
| Site Supervisor | Matthew Monaghan |  | +61498112738 |
| OHS Manager | Cameron Jackson |  | +61 4 12286008 |

Description of work to be done

| JOB START DATE | JOB END DATE | TYPE OF WORK TO BE DONE | DESCRIPTION OF WORK |
| --- | --- | --- | --- |
| 14/09/2024 | 14/09/2025 | Construction of a new dwelling | Split level dwelling |

This Health and Safety Coordination Plan (HSCP) must be reviewed if there are any significant changes to the work. It must be available for inspection by anyone doing work on the project. All employees and contractors are responsible for their own safety, their own employees and those who may be affected by their work.

All visitors must be inducted into this site, or accompanied by an inducted or trained person while here.

Roles and Responsibilities

The principal contractor is responsible for:

Establishing and displaying signage stating the principal contractor details for the construction project

The preparation and implementation of a HSCP for the construction project, including all associated policies and procedures Informing all persons of the HSCP prior to the start of works

Reviewing and revising the content of the HSCP to ensure that it remains up to date

Obtaining Safe Work Method Statements (‘SWMS’) prior to the start of any High Risk Construction Works onsite Establishing arrangements to ensure compliance with specific requirements such as facilities and amenities onsite Managing the risk associated with construction materials, waste, plant, traffic and essential services

Being the designated site occupational health and safety (OHS) contact when the site supervisor is unavailable or offsite

PRINCIPAL CONTRACTOR

The site supervisor is responsible for:

Coordinating the safe interaction between contractors and employees onsite; Ensuring that this HSCP is available for sighting

Ensuring the Principal Contractor signage is posted and visible

Consulting with employees and contractors on OHS matters that may affect them Coordinating SWMS amendments as appropriate

Ensuring all contractors and visitors to the site receive the relevant site safety induction

Confirming that contractors have established systems to provide site safety briefings to their employees Confirming that site First Aid requirements are established and monitored

SITE SUPERVISOR

OHS Managers are responsible for:

Facilitating consultation between the principal contractor, site supervisor or any other person to manage the risk associated with hazards that may impact the health and safety of employees onsite;

Participating in the completion of site reviews (where required) Participating in the completion of incident investigations (where required)

OHS MANAGER

Contractors engaged by the principal contractor are responsible for:

Identifying all high risk construction work associated with their activities and ensuring SWMS are developed and implemented Complying with the duties as listed under ’Employees’

Adhering to all safety policies, procedures and site rules at all times whilst onsite; Complying with the details of this HSCP

Complying with any direction given to them by the principal contractor

Undertaking a site-specific induction prior to the commencement of works and signing a written acknowledgement that they have completed this induction

CONTRACTORS

EMPLOYEES

Employees are responsible for:

Taking reasonable care of their own health and safety whilst onsite

Taking reasonable care to ensure that their conduct does not adversely affect others Complying with all instructions or directions onsite, whether written or verbal

# High Risk Construction Work (HRCW)

Prior to the commencement of any high risk construction work, a Safe Work Method Statement (SWMS) must be compiled and submitted to the pincipal contractor.

Identify the work that is high risk construction work

Specify hazards relating to the high risk construction work and risks to health and safety associated with those hazards; Describe the measures to be implemented to control the risks;

Describe how the control measures are to be implemented, monitored and reviewed.

The SWMS must:

All SWMS will be assessed by a competent person nominated by the Principal Contractor to ensure they identify all hazards associated with high risk construction work and the associated risk controls to be adopted for each identified hazard.

Working at heights where there is a risk of a person falling more than two metres

The following high risk construction work must be identified and requires a Safe Work Method Statement (SWMS) before work commences:

# Notifiable Work

Intention to perform construction excavation work

The following notifiable work has been identified:

All notifications will be submitted within the appropriate timeframe to WorkSafe Victoria following the requirements outlined on their website

# Hazard ID and Risk Management

We have identified the hazards and associated risks to health and safety, where possible we will eliminate the risk, if we can’t eliminate the risk then we will put in place controls to minimise by substitution, isolation, or implementing engineering controls. If the risk still remains it will be further minimised using administration controls and PPE.

The hazards and risk controls for this work will be discussed with employees, contractors and all visitors as part of their induction to the site.

All risk controls fall into two categories: ELIMINATION and MINIMISATION.

ELIMINATION: This involves removing the risk to health and safety from the work site. It is the most effective risk control and should always be prioritised.

MINIMISATION: When a risk can not be removed, it should be minimised. This can be done using the following controls: substitution, isolation, engineering, administration and PPE. These controls are explained below.

Using personal protective equipment to protect against harm

PPE

Using safe methods of work, processes or procedures

ADMINISTRATION

Using physical control measures

ENGINEERING

Separating people from the hazard

ISOLATION

Using a less hazardous thing, substance or work practice

SUBSTITUTION

Avoiding the sources of harm

ELIMINATION

All risk controls are ranked in order of most effective (elimination) to least effective (PPE)

HIERARCHY OF CONTROLS

# Known Hazards and Risks

This section lists each known or expected hazard, the associated risks and the good practice controls to manage these risks. The following controls shall be established at all times:

| Controls |  |
| --- | --- |
| Secure our work site from the public | ISOLATION |
| Apply the relevant legislation, guidelines or manufacturers instructions so far as reasonably practicable | ADMINISTRATION |
| Use a Hazard Board to list all current hazards, risks and controls on site | ADMINISTRATION |
| Ensure all contractors and employees are trained and competent in the task being carried out | ADMINISTRATION |
| Have appropriate emergency response procedures in place for the high risk construction works on site | ADMINISTRATION |
| Enforce that the required PPE is worn at all times | PPE |

| Known hazards | Risks created by known hazards | Risk controls that will be put in place when work commences |
| --- | --- | --- |
| Contractors and visitors | Visitors/children to work site being exposed to all work site hazards that could result in injury, illness or death | ELIMINATION No unsupervised children permitted on site ISOLATION Secure perimeter fencing established around the site ADMINISTRATION Escort/supervision of any visitors/contractors/clients on site ADMINISTRATION Induct all visitors/contractors/clients to site ADMINISTRATION All employees/contractors to sign in/out on entry and exit to the site ADMINISTRATION Use hazard boards and induction to warn visitors of hazards |

| Outbreak of infection/illness | Exposure to large scale outbreaks of infectious diseases (or similar) which cause illness, health complications and in some cases death | ISOLATION If anyone is unwell or travelled overseas recently they are not to enter the site ADMINISTRATION Follow physical interaction guidelines between workers (employees and contractors) e.g. through physical distancing, split shifts, staggered meal breaks, flexible working arrangements, separate vehicles ADMINISTRATION Ensure adequate site facilities are available including toilets and handwashing ADMINISTRATION Follow basic hygiene levels which includes: thorough hand washing and drying with soap and paper towels or sanitiser, not touching your face, sneezing or coughing into your elbow or tissue ADMINISTRATION Carry out frequent cleaning of work surfaces with disinfectant (in particular high-touch surfaces) and equipment/tools ADMINISTRATION Set up a contactless delivery or managed entry (inside and outside premises) ADMINISTRATION Keep a record of all workers (employees and contractors) and visitors entering and leaving the site PPE Do not share PPE and follow instructions to limit the spread of infection eg remove gloves inside out and dispose of correctly |
| --- | --- | --- |
| Slips and trips | Exposure of employees and others to same level falls that could result in injury. e.g. breaks, cuts, abrasions etc. | ISOLATION Access/egress ways to remain clear of obstruction at all times ISOLATION Provision of bins for waste ISOLATION Storage of building materials in designated areas ADMINISTRATION Promotion of high level housekeeping practices |
| Moving vehicles | Exposure of employees to being struck by vehicles that could result in injury or death. e.g. crush, breaks, cuts, abrasions, etc. | ISOLATION Provision of designated loading/unloading areas onsite ADMINISTRATION Establish a Site Traffic Management Plan ISOLATION Use of barriers to isolate mobile plant and pedestrians onsite ADMINISTRATION Active 3 metre exclusion zone around all mobile plant ADMINISTRATION Use of spotters during movement of mobile plant where visibility is restricted ADMINISTRATION Ensure manufacturer’s log books are completed prior to use and commercial driving laws are followed |

| Electricity and electrical appliances | Exposure of employees to electricity that could result in injury or death through electric shock or electrical burns. | ISOLATION Use Lock Out/Tag Out practices including testing for ‘dead’ (where applicable) ENGINEERING Use approved RCDs ADMINISTRATION Equipment, leads and RCDs to be checked by a competent person ADMINISTRATION All RCDs must be interval tested and tagged accordingly ADMINISTRATION Power boards not to be used in series |
| --- | --- | --- |
| Hand and power tools in use | Exposure of employees (through incorrect tool use, vibration or equipment failure) that could result in injury or death. e.g. serious laceration, puncture wounds, nerve damage, cuts and abrasions. | ENGINEERING Ensure guards remain in place at all times during operation ADMINISTRATION Use correct equipment for the task ADMINISTRATION Ensure regular equipment checks and servicing |
| Lifting or manual handling materials and equipment | Exposure of employees (through incorrect tool use, vibration or equipment failure) that could result in injury or death. e.g serious laceration, puncture wounds, nerve damage, cuts and abrasions. | SUBSTITUTION Use mechanical aids e.g. forklifts, pallet jack, etc ADMINISTRATION Use correct lifting techniques and ask for help if needed ADMINISTRATION Ensure staff are trained in manual handling |
| Noise | Exposure of employees to high noise levels (above 85dB) that could result in illness. e.g, hearing loss. | ISOLATION Ensure exclusion zones are in place and other employees/visitors keep clear ADMINISTRATION Monitor noise levels on site ADMINISTRATION Completion of audiometric testing (2 yearly) on employees exposed to noise in excess of 85dBA PPE Wear hearing protection in designated areas on site PPE Ensure compliance with hearing protection usage |
| Dust | Exposure of employees to dust that could result in illness. e.g, lung disease. | ENGINEERING Use extraction systems on tools that create dust ENGINEERING Dampen down work areas to reduce dust disturbance PPE Wear appropriate mask/respirator and ensure usage ADMINISTRATION Monitor employees health for exposure to dust ADMINISTRATION Keep work areas tidy to reduce dust |
| Adverse weather conditions | Exposure of employees to weather elements resulting in injury or illness. | ELIMINATION Postpone all working at heights and lifting operations ISOLATION Secure tools, materials and equipment PPE Wear clothing appropriate to the weather conditions |

| Ladders | Exposure of employees to falls from height that could result in injury or death. | ELIMINATION Consider if work can be undertaken from the ground using extension tools ELIMINATION Consider if an elevating work platform such as a scissor lift, cherry picker, or a fixed or mobile work platform can be used ELIMINATION Consider if scaffolding or use of a mobile scaffold can be used ELIMINATION Consider if a step platform can be used ADMINISTRATION Ladders for short duration work or access to the work area only ADMINISTRATION Ladders to be inspected for defects prior to use ADMINISTRATION Ensure regular maintenance checks by a competent person ADMINISTRATION Ladder to be set up correctly 4:1 ratio, secure base and tie off top ADMINISTRATION Top of the ladder to be secured in position by hooks, chains, ropes or other approved means ADMINISTRATION Ladder to be placed on level firm footing and lashed/clamped ADMINISTRATION Do not work off the top two rungs/steps |
| --- | --- | --- |
| Risk of fire | Exposure of employees to burns that could result in injury or death. | ADMINISTRATION All hot work to be conducted via permit only. The employee who is in direct control of the work must ensure that one or more Fire Watch persons are present for the entire duration of the hot work ADMINISTRATION Keep clear of combustible materials when using tools ADMINISTRATION Fire watch to conduct inspections of area for 60 minutes after completion of hot work. Periodic inspections to be conducted for an additional 3 hours after the 60 minute period ADMINISTRATION Fire extinguisher/s available and staff trained in use ADMINISTRATION Ensure an emergency response plan is in place before starting work |

| Fatigue | Exposure of employees to long work hours and repetitive work tasks that could result in injury or illness. e.g, Lapse in concentration due to fatigue. | ADMINISTRATION Monitor staff for signs of fatigue ADMINISTRATION Ensure excessive shift lengths (e.g. 12 hours plus) and continuous work days without rest are kept to a minimum ADMINISTRATION Reduce the time employees are required to perform physically and mentally demanding work ADMINISTRATION Minimise work periods during extreme temperature ENGINEERING Provide adequate facilities for rest and meal breaks ADMINISTRATION Encourage regular breaks, in-take of fluids and food ADMINISTRATION Encourage staff to inform you if they are taking medication that can cause drowsiness |
| --- | --- | --- |
| Stacked/Stored materials | Exposure of employees to unstable materials that could result in injury or death. e.g, crush injuries. | ISOLATION Contain building materials in designated storage areas ISOLATION Secure all stacked material appropriately ADMINISTRATION Keep materials clear of access ways ADMINISTRATION Use bins for waste ADMINISTRATION Ensure site is clean and tidy |
| Working alone | Exposure of isolated employees to risks that could result in injury or death. e.g suspension trauma after a fall, vehicle crash and no one to raise alarm, etc. | ELIMINATION Avoid working alone whenever possible ELIMINATION No High Risk Construction Work (HRCW) to be undertaken alone ENGINEERING Use automatic warning devices, e.g. panic alarms ADMINISTRATION Provide appropriate training and education to lone employees ADMINISTRATION Establish a check-in procedure and ensure regular contact is maintained with employees ADMINISTRATION Establish ways to account for people (visually or verbally) while they are working ADMINISTRATION Ensure higher risk tasks to be done when another employee can be present and provide help |

| Drugs and alcohol | Exposure of employees and others to intoxicated employees that could result in injury or death. e.g. poor concentration, carelessness, risk-taking behaviour and errors in judgement | ELIMINATION Remove any intoxicated employees from site ADMINISTRATION Monitor employees for signs of impairment ADMINISTRATION Ensure drug/alcohol policy shared with employees ADMINISTRATION Discuss dangers of drug/alcohol use with employees and encourage safe working practices ADMINISTRATION Implement random and event based drug testing |
| --- | --- | --- |
| Animals | Exposure of employees to animals that could result in injury or illness. e.g, bites, disease. | ELIMINATION Ask clients/visitors to keep pets away from site ISOLATION If pets are present or brought to site they must be controlled at all times (e.g. dogs tied up) ADMINISTRATION Keep clear of stray animals ADMINISTRATION Call local animal control authority if concerned about an animal's behaviour |

# Project Specific Hazards/Risks

The section lists additional hazards and risks and confirms the chosen controls for the task present. Where possible we will eliminate the risk to health and safety, if we can’t eliminate the risk then we will put in place controls to minimise by substitution, isolation, and implementing engineering controls. If the risk still remains it will be further minimised using administration controls and PPE. At all times we will implement the following controls:

Adhere to the relevant legislation, guidelines or manufacturers instructions Use a Hazard Board to list all current hazards, risks and controls on site Ensure all employees are trained and competent in the task being carried out Secure our site from the public

Restrict persons under the age of 15 from accessing the site Enforce that the required PPE is worn at all times

Have appropriate emergency procedures in place and a current SWMS for the high risk construction work (HRCW) on site

| Identified hazard | Risk created by hazard | Confirmation of task | Risk controls that will be put in place |
| --- | --- | --- | --- |
| Working at height - This includes roof work, any work off a scaffold or out of an MEWP (more than 2 metres) | Exposure to falls from height more than 2 metres that could result in injury or death. | We will be working at height, greater than 2 metres | ISOLATION Fence off underneath the work area ISOLATION Isolate the hazard using methods such as scaffolding, guarded work Platforms, MEWPS and edge protection ENGINEERING Use catchment methods such as netting to catch tools and debris ADMINISTRATION Working at height equipment must be fit for purpose and in good condition PPE Fall restraint equipment to be used eg harness |
| Erecting or dismantling scaffolding (portable or fixed) | Exposure to falls from height more than 2 metres that could result in injury or death. | We will be erecting, dismantling or working on scaffolding | ADMINISTRATION To be installed and tagged by a competent installer per AS/NZs 1576 ADMINISTRATION A handover certificate is provided when erecting complete and following any alterations. The certificate should be kept on site. Regularly checked (not exceeding 30 days) as safe for use and current tag is on display ADMINISTRATION Ensure scaffold is fit for purpose eg correct load rating |

| Identified hazard | Risk created by hazard | Confirmation of task | Risk controls that will be put in place |
| --- | --- | --- | --- |
| Conducting excavations - This includes any work that may come in contact with overhead/underground utilities | Exposure of employees to open or unstable trenches or excavated faces that could result in injury or death, e.g., engulfment, suffocation or exposure of employees to gas, electricity or sewerage during digging that could result in injury, illness or death, e.g., explosion, electrocution and disease. | There will be excavations on site | ISOLATION Cover/fence/barricade holes ISOLATION All services to be identified prior to the commencement of work, implement lockout/tagout and de-energisation process to ensure safe isolation ENGINEERING Use shoring for excavations greater than or equal to 1.5m or in unstable ground ADMINISTRATION Keep other employees/visitors clear and stay visible to operator ADMINISTRATION Ensure regular equipment servicing ADMINISTRATION Dig in appropriate weather conditions ADMINISTRATION Inspect the excavation daily before work starts or after any adverse weather conditions |
| Using heavy machinery or cranes | Exposure of employees to machinery and their components or vehicles that could result in injury or death, e.g., degloving, crush, breaks, cuts, abrasions, etc. | There will be heavy machinery on site | ADMINISTRATION When not in use ensure all plant, machinery and equipment are secure and ensure no unauthorised use ADMINISTRATION Keep clear of swing radius ADMINISTRATION Bucket and attachments lowered before approach ADMINISTRATION Stay visible to operator ADMINISTRATION Regular inspections/servicing, daily pre-starts to be conducted ADMINISTRATION Do not exceed weight limits ADMINISTRATION Up-to-date SWMS available on site ADMINISTRATION A plant, machinery and equipment register must be up-to-date and available on site |

| Identified hazard | Risk created by hazard | Confirmation of task | Risk controls that will be put in place |
| --- | --- | --- | --- |
| Working with concrete | Exposure of employees to multiple risks that could result in injury, death, e.g., hazardous substances, moving vehicles, exploding pipes, falling objects, etc. | We will be working with concrete | ISOLATION Do not work above reinforcing steel ENGINEERING Cap all exposed reinforcing steel ADMINISTRATION Eye wash facilities must be available on site ADMINISTRATION Ensure regular equipment servicing ADMINISTRATION Complete a hazardous substance register ADMINISTRATION Ensure Safety Data Sheets (SDS) are available on site |
| Carrying out any hot works (welding, brazing) | Exposure of employees to hot gases that could result in injury or death, e.g., explosion and burns. | We will be carrying out hot works | ISOLATION Clear area of all combustible materials ISOLATION Secure all gas bottles ADMINISTRATION Clean water and fire extinguisher must be available at all times ADMINISTRATION Up-to-date SWMS available on site ADMINISTRATION A permit to work process is required ADMINISTRATION All hot work to be conducted via permit only. The employee who is in direct control of the work must ensure that one or more Fire Watch persons are present for the entire duration of the hot work ADMINISTRATION Fire watch to conduct inspections of area for 60 minutes after completion of hot work. Periodic inspections to be conducted for an additional 3 hours after the 60 minute period |

| Identified hazard | Risk created by hazard | Confirmation of task | Risk controls that will be put in place |
| --- | --- | --- | --- |
| Collapsing floor/structure | Exposure of employees to restricted spaces that could result in injury or illness, e.g., entrapment, heat exhaustion, disease contraction, etc. | We will be using temporary supports | ENGINEERING Use approved props installed to engineer's design ADMINISTRATION Ensure props are in good condition |
| Handling glass | Exposure of employees to broken glass that could result in injury or death, e.g., lacerations. | We will be handling glass | ISOLATION Area where works to be undertaken to be isolated as an exclusion zone ISOLATION Do not work underneath where glass is being installed ADMINISTRATION Glass to be disposed of correctly ADMINISTRATION All broken glass to be cleaned up immediately |
| Working in cramped conditions (underfloor, roof spaces) | Exposure of employees to collapsing structures that could result in injury or death, e.g., crush, asphyxiation. | There will be work in cramped conditions | ELIMINATION Do not enter areas not large enough to comfortably exit ADMINISTRATION Use a standby person ADMINISTRATION Ensure sufficient ventilation and lighting |

All employees on this job are required to wear/use the following PPE, during their work on our work site. Training in its care, maintenance and use will be provided. The PPE to be worn when required on this work site is:

Steel cap shoes/Boots	Eye protection

Gloves	Hearing protection

High vis clothing	Dust mask

Hard hat	Fall protection (harness systems)

PERSONAL PROTECTIVE EQUIPMENT (PPE)

# Site Reviews

Fortnightly site reviews will be conducted and recorded using the HazardCo App.

# Toolbox Meetings

Weekly toolbox meetings will be conducted and recorded using the HazardCo App. All employees and contractors that are present on the site during a toolbox meeting are expected to attend and are encouraged to provide input on matters that affect safety for this project. All attendees will be captured through a photo in the HazardCo App or listed in a record of attendance.

# Training, Licensing and other Qualifications

Construction induction training

All employees carrying out any work on site are to have construction induction training and be the holder of a Construction Induction Card (White Card), Red Card or other equivalent recognised induction card.

Training needs will be identified and provided by the company or the individual contractor for all employees, including management, as necessary. This training will range from induction training, on-going health and safety training and any specialist training that may be required.

# Health and Safety Training

Where licensing, certification or other similar recognition of training is required, details of licensing or certification will be reviewed.

# Site induction training

All employees carrying out any work on site are to complete a site induction prepared by the Principal Contractor. All visitors to the site are to receive a basic site induction advising them of any safety requirements that may affect them.

# Refresher training

Refresher training (as required) will be provided to all employees to enable them to perform their tasks safely.

CONTRACTORS

Each of our contractors (employers) are expected to manage their own active health and safety management system. The competence of our contractors has been assessed or reviewed

e.g. they have the appropriate certification, licensing, qualifications and competency for the job they are undertaking. Examples of specialist contractors that will be asked to provide appropriate evidence include:

Electrical Contractors Electricians Plumbers  Scaffolders

Dogmen and riggers Crane operators

Mobile concrete placing boom operators

Forklift operators

Tele-handler operators Hoist and MEWP operators MEWP operators

Earthmoving equipment operators Welders

Asbestos removalists

*uncertified or unqualified trainees or apprentices will need to be under the direct supervision of a qualified person or supervisor.

All contractors will be inducted into the site as outlined in our induction procedures.

# Plant, machinery and equipment

All plant, machinery and equipment on this site will be in good working order, with all safety mechanisms intact. Where applicable, machinery will be operated and maintained in accordance with licensing requirements, manufacturer's instructions or Standard Operating Procedures (SOP). If any equipment is not in safe working order, it will be removed from service until maintenance is complete, or repaired by a competent person, before continued use.

All plant, machinery and equipment will hold records of pre-start and be checked regularly and have an available SWMS.

# Health and Safety Coordination Plan Acceptance

This document identifies hazards for the work we are undertaking and actively managing. Risks may change according to these hazards and as such, so may our controls. Changes will be documented at the time and communicated appropriately. This plan outlines the minimum requirements for the appropriate risk controls, however, where additional risk controls are required the supporting documentation (SWMS etc) should be provided.

HSCP prepared by:

| Signature: | Date: |
| --- | --- |

HSCP reviewed by:

| Signature: | Date: |
| --- | --- |

Safety Rules

| The following rules are specific to this site and should be followed or adhered to by all employees and contractors. Contact the Supervisor for further questions or issues: |  |
| --- | --- |
| All workers (employees and contractors) to comply with all health and safety requirements on-site SWMS to be completed for all High Risk Construction Work Licences required for high risk work No high risk work to be undertaken alone Report all incidents to the supervisor All workers to have white/red cards Safety signs are to be followed Excavations over 1.5 metres deep are to be shored, shielded or battered Follow relevant No Go Zone / Danger Zone clearance requirements when working near overhead power lines. Keep two steps from the top of a stepladder and three steps from the top of a straight ladder Ladders to be secured at a slope of 4:1 Ladders to be industrial and 120kg Safe Working Load Harnesses only used for minor work Provide fall protection for work over 2 metres Only licenced scaffolders to erect, alter or dismantle scaffolds over four metres high Don't remove guard rails or scaffold components unless authorised Mobile plant operators to have appropriate qualifications | Mobile plant to have warning light and alarm, and service records Hi-vis vests worn around mobile plant Electrical leads to be tested and tagged every three months RCDs to be tested and tagged regularly Electrical leads to be up off the ground in a tidy manner 10 amp leads not to exceed 35 metres high Industrial multi-plug devices to be used All hazardous substances added to the register and provide access to SDS details Fit flashback arrestors to gas hoses Wear PPE when welding or gas cutting Use vacuum and wear PPE if cutting MDF Dry cutting, grinding or polishing silica containing products must not be undertaken without adequate engineering controls and Respiratory Protective Equipment. Wear appropriate PPE at all times No harassment, bullying, discrimination, vandalism or bad behaviour No illegal drugs or alcohol on-site Follow SunSmart clothing requirements Use correct manual handling procedures Keep your work area clean and tidy No children or animals allowed on-site Do no walk through barricaded areas or exclusion zones |

Emergency Response Plan

Emergency Situations

In an emergency notify the Principal Contractor, Supervisor, First Aider or Health and Safety Representative (if applicable) immediately and make the injured person/s comfortable and have them initially treated by a First Aider.

The following procedures are in place for any potential emergencies that could arise in our site. All Notifiable Events will be notified to WorkSafe Victoria on 13 23 60 immediately.

Shut down any plant, machinery or equipment.

Provide first aid if someone is injured and stay with them.

Evacuate the site, move to the safe assembly area, ONLY when safe to move Dial 000 and ask for an ambulance.

Arrange for someone to meet the ambulance at the front of the site.

Report Incident details to Principal Contractor, Supervisor, and OHS Manager as advised.

ACCIDENT PROCEDURES

| Site supervisor: | Site supervisor contact number: |
| --- | --- |
| Matthew Monaghan | +61498112738 |

| First aider/s: | First aider/s contact number: |
| --- | --- |
| Matt Monaghan | +61498112738 |

| Fire warden/s: | Fire warden/s contact number: |
| --- | --- |
| Camerson Jackson | +61412286008 |

| First Aid Kit Location: | Our emergency signal is: |
| --- | --- |
| Carried by Contractor | Verbal Communication |

| Assembly point: | Nearest medical centre: |
| --- | --- |
|  | Sunbury Medical Centre |

| Fire fighting equipment details: | Other emergency procedures: |
| --- | --- |
| Carried by Contractor |  |

Contact emergency services on 000 if required - nearest doctor and hospital to be posted on site Arrange for any injured person to be transferred to the nearest doctor or hospital if required Isolate/preserve the incident scene and make safe

Carry out an emergency evacuation if necessary

Contact the Principal Contractor or WorkSafe VIC (132 360) and any other relevant authorities for serious incidents Link: https://www.worksafe.vic.gov.au/report-incident-criteria- reportable-incidents (https://www.worksafe.vic.gov.au/report-incident-criteria-reportable-incidents)

Complete all incident reports as required.

Site supervisor to complete the following: